# Extended Deep Dive: EDA Tools and HPC in Semiconductor Design

## 1. The Expanding Scope of EDA

EDA is no longer limited to basic logic and layout; modern semiconductor design has expanded significantly:
• Advanced Node Challenges:
  - At 5nm and below, variability, double/triple patterning, and electro-migration need complex modeling.
  - Includes process variation, IR-drop maps, electro-thermal co-simulation.
• Heterogeneous Integration:
  - Chiplets and advanced packaging (2.5D/3DIC) require co-design of package, die, and board.
  - Tools like Cadence Integrity 3D-IC and Synopsys 3DIC Compiler rely on HPC for combined thermal/timing analysis.
• Hardware/Software Co-Design:
  - Software bring-up and verification begin pre-silicon with emulation and hybrid simulation.
  - Emulators like Cadence Palladium Z2 and Synopsys ZeBu integrate with HPC for massive regressions.

## 2. HPC Requirements Driven by EDA

EDA workloads stress compute, memory, and I/O differently depending on the stage:

RTL Simulation: Tens of thousands of small jobs; modest CPU per job; high concurrency and metadata I/O.
Synthesis: Medium jobs; CPU-bound, benefits from high clock speeds and >32 cores; moderate I/O.
Place & Route (P&R): Large jobs; memory-intensive (hundreds of GB); heavy random I/O to large databases.
STA (Sign-off): Many small/medium jobs; embarrassingly parallel across corners/modes; moderate I/O.
DRC/LVS: Large single jobs; need 1000–5000 cores for big designs; heavy sequential and random I/O to TB-scale GDSII.
Parasitic Extraction: Large solver-driven jobs; benefit from CPU+GPU acceleration; large mesh file I/O.
Thermal/IR Analysis: Solver-heavy; scales with cores and GPUs; moderate I/O.

## 3. HPC Architectural Patterns in Semiconductor EDA Farms

Compute Layer:
• Mix of CPU (e.g., 2×64-core EPYC, 1–2 TB RAM) and GPU-enabled nodes (A100/H100 or Gaudi2/3).
• Hybrid clusters (on-prem + cloud burst) for peak demands.

Scheduler Layer:
• License-aware scheduling critical for floating licenses (FlexLM).
• Job arrays and dependency chains orchestrate multi-stage flows.

Storage Layer:
• Parallel file systems (Lustre, GPFS, BeeGFS) for shared design databases.
• NVMe tiered storage for scratch/work directories to reduce I/O bottlenecks.
• Object storage (e.g., S3) for archiving intermediate data and waveform dumps.

Network Layer:
• InfiniBand HDR/NDR (200-400 Gbps) for low-latency MPI-based distributed solvers.
• RoCEv2 used in cloud environments with careful tuning.

Cloud Integration:
• Cloud bursting (AWS EC2 C7i/Hpc7g, OCI BM.GPU4, Azure HBv4) during peak workloads.
• Synopsys Cloud and Cadence CloudBurst handle hybrid licensing and submissions.

## 4. Performance and Scaling Considerations

• Scaling Behavior: Most EDA workloads scale sub-linearly; focus is on throughput (many jobs in parallel).
• Memory Sensitivity: Sign-off tools often need >512 GB per job; hierarchical partitioning demands fast inter-node communication.
• Checkpoint/Restart: Essential for multi-day jobs to prevent restart on node failure.
• I/O Optimization: Use of read-only design database replicas and per-job NVMe scratch reduces contention.

## 5. Real-World Metrics from Semiconductor Companies

• Top-tier CPU nodes: 64–128 cores/node, 1–2 TB RAM.
• Cluster size: 10k–100k cores in on-prem EDA farms.
• Typical regression suite: 1–2 million tests/night across ~50k cores.
• Peak tape-out window: 2–3× baseline compute demand, often bursts to cloud.
• Storage needs: 5–50 PB with sustained 100–200 GB/s I/O throughput.
• Network latency target: <2 µs for MPI-based distributed solvers.

## 6. Emerging Trends

• AI in EDA: AI-driven floorplanning, auto-routing, test generation (e.g., Synopsys DSO.ai) accelerated on GPUs.
• 3DIC and Thermal-Aware Sign-off: Computationally intensive due to TSV modeling and stacked-die effects.
• Cloud-Native EDA Workflows: SaaS models with elastic cloud HPC compute.
• Green HPC: Focus on energy-efficient clusters with liquid cooling and optimized scheduling.

## Key Takeaways

• EDA complexity is outpacing Moore’s Law, demanding HPC scale to keep up.
• HPC is the production engine for chip design, enabling faster tape-out and higher yield.
• Hybrid (on-prem + cloud) HPC is standard for balancing cost and scalability.